PHYSICS 1.3						Name:

WORKSHEET THREE 							RESISTANCE

1.	Describe what is meant by the resistance of a conductor.

2.	List the four factors that affect the resistance of a wire.

(i)

(ii)

(iii)

(iv)

3.	Explain the difference between an ohmic and a non-ohmic conductor.

4.	Calculate the resistance of a 12 Volt car stereo that carries a current of 4 A.

12 Ω

18 Ω

. . .

A

V

5.	The circuit shown has two resistors. One is 12 Ohms and the

other is 18 Ohms. The voltmeter reads 6 Volts.

- Calculate the current in the 12 Ohm resistor.

- Calculate the voltage across the 18 Ohm resistor.

- Calculate the voltage of the battery.

- Calculate the current shown by the ammeter if the battery was changed to 24 V.

6.	In an Ohm’s law experiment, the voltage across a resistor was measured at the same time as 	the current through it. The results are shown in the chart.

| Voltage (V) | 0 | 2 | 4 | 6 | 8 | 10 | 12 | 14 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Current (A) | 0 | 1.6 | 3.0 | 4.2 | 5.2 | 6.0 | 6.6 | 7.0 |

(a)	Draw a graph of these results below (put voltage on the y axis).

- From your graph, what is the current when the voltage is 9V?

- What is happening to the slope of the graph?

- Explain what is happening to the resistance in this experiment.

- What variable must be controlled in order for this resistor to obey Ohm’s law?

Circuit 1

. .. . . .. .

7.	The battery, bulbs and resistors are identical in the two circuits on the right.

Which circuit offers the lowest total resistance? Explain your answer.

Circuit 2

. . .